# Sentinel-X:

# Secure AI-Powered Wi-Fi Indoor Positioning and Intelligent Asset Tracking System

By:

Muhammad Awais Ali

53015

Abdulahad

47374

Supervised by:

Syed Yawar Abbass Zaidi

Faculty of Computing

Riphah International University, Islamabad

Fall 2026

A Dissertation Submitted To

Faculty of Computing,

Riphah International University, Islamabad

As a Partial Fulfillment of the Requirement for the Award of the Degree of

Bachelors of Science in Cyber Security

Faculty of Computing

Riphah International University, Islamabad

Date: [date of final presentation]

Final Approval

This is to certify that we have read the report submitted by name of student(s) (CMS #), for the partial fulfillment of the requirements for the degree of the Bachelors of Science in Computer Science (BSSE). It is our judgment that this report is of sufficient standard to warrant its acceptance by Riphah International University, Islamabad for the degree of Bachelors of Science in Computer Science (BSSE).

Committee:

| 1 | [Name Supervisor] (Supervisor) |
| --- | --- |
| 2 | [Name of HOD/chairman] (Head of Department/chairman) |

Declaration

# We hereby declare that this document “Sentinel-X: Secure AI-Powered Wi-Fi Indoor Positioning and Intelligent Asset Tracking System” neither as a whole nor as a part has been copied out from any source. It is further declared that we have done this project with the accompanied report entirely on the basis of our personal efforts, under the proficient guidance of our teachers, especially our supervisor Syed Yawar Abbas Zaidi. If any part of the system is proved to be copied out from any source or found to be reproduction of any project from anywhere else, we shall stand by the consequences.

___________________________

Muhammad Awais Ali

53015

___________________________

Abdulahad

47374

Dedication

Insert dedication here…

Acknowledgement

First of all we are obliged to Allah Almighty the Merciful, the Beneficent and the source of all Knowledge, for granting us the courage and knowledge to complete this Project.

[Students will acknowledge here anyone who has helped in the project. It can include Supervisor(s), Teachers, Classmates, Friends and Family]

___________________________

Muhammad Awais Ali

53015

___________________________

Abdulahad

47374

Abstract

Indoor positioning has become increasingly important in environments such as universities, laboratories, hospitals, and offices where GPS signals are unreliable or unavailable. Existing indoor positioning systems often suffer from high deployment costs, limited scalability, inadequate security, or poor localization accuracy due to fluctuating Wi-Fi signals. This project proposes Sentinel-X, a secure AI-powered Wi-Fi Indoor Positioning and Intelligent Asset Tracking System that utilizes ESP32-based tracking tags and anchor nodes to estimate the real-time location of indoor assets and authorized users.

The system leverages Wi-Fi RSSI measurements combined with localization techniques such as fingerprinting, RSSI calibration, Kalman filtering, and trilateration to improve positioning accuracy. Artificial Intelligence algorithms will further enhance localization performance and detect abnormal movement patterns, while cybersecurity mechanisms including encrypted communication, secure authentication, replay protection, and spoofing detection will protect the system from common wireless attacks. A web-based dashboard will provide administrators with real-time monitoring, location visualization, and movement history.

The proposed solution aims to deliver an affordable, scalable, and secure indoor positioning system suitable for educational institutions and similar environments. Sentinel-X combines IoT, Artificial Intelligence, Wireless Networking, and Cybersecurity into a unified platform capable of improving indoor asset management and security.

# Introduction

Indoor positioning systems have become an essential component of modern smart environments where accurate location information is required for asset management, security monitoring, and operational efficiency. While the Global Positioning System (GPS) performs effectively in outdoor environments, it cannot provide reliable positioning indoors due to signal attenuation caused by walls, ceilings, and other physical obstacles.

Various indoor positioning technologies such as RFID, Bluetooth Low Energy (BLE), Ultra-Wideband (UWB), and Wi-Fi have been developed to address this challenge. Among these, Wi-Fi-based localization has gained significant attention because it utilizes existing wireless infrastructure, making it a cost-effective and scalable solution. However, traditional Wi-Fi localization systems often experience fluctuating RSSI values, reducing localization accuracy.

Furthermore, many existing systems lack integrated cybersecurity mechanisms, making them vulnerable to spoofing, replay attacks, and unauthorized access.

Sentinel-X addresses these challenges by combining Wi-Fi-based localization with Artificial Intelligence and Cybersecurity to develop a secure, intelligent, and affordable indoor positioning system capable of real-time asset and personnel tracking.

## Goals and Objectives

- Goal

To develop a secure AI-powered Wi-Fi indoor positioning system for accurate real-time tracking of assets and authorized users in indoor environments.

- Objectives

- Design a Wi-Fi-based indoor positioning system using ESP32 tracking tags and anchor nodes.

- Estimate indoor locations using RSSI-based localization techniques.

- Improve localization accuracy through AI-assisted signal processing.

- Secure communication using authentication, encryption, and replay protection mechanisms.

- Detect abnormal asset or user movement using Artificial Intelligence.

- Develop a web-based dashboard for real-time monitoring and management.

## Scope of the Project

The proposed system focuses on developing a prototype for secure indoor positioning within a university laboratory or similar indoor environment.

- Wi-Fi-based indoor positioning

- ESP32 tracking devices

- RSSI-based localization

- Artificial Intelligence for localization improvement

- Cybersecurity mechanisms

- Web-based monitoring dashboard

It covers these main areas:

- IoT Hardware

- ESP32 tracking tags

- ESP32 anchor nodes

- Wi-Fi/RSSI data collection

- Indoor Positioning

- Collect RSSI from multiple anchors

- Process the RSSI data

- Estimate the real-time position of an asset

- AI/Processing

- Kalman Filter for RSSI smoothing

- KNN/fingerprinting for location estimation

- Isolation Forest for detecting abnormal movement

- Communication

- MQTT for transferring data from the ESP32/anchors to the backend

- Web Dashboard

- Real-time asset location

- Asset  status

- Movement history

- Alerts

- Security

- Authentication

- Secure MQTT communication

- Encryption/TLS

- Anomaly detection

- Database

- Store asset information

- Location history

- Logs and relevant system data

# Literature Review

Numerous indoor positioning systems have been proposed using Wi-Fi, Bluetooth Low Energy (BLE), RFID, and Ultra-Wideband (UWB) technologies[1]. Wi-Fi-based localization has attracted significant attention because it utilizes existing network infrastructure, making it more economical than RFID or UWB solutions. However, RSSI measurements are highly sensitive to environmental changes such as walls, furniture, and human movement, which reduce localization accuracy. Recent studies have addressed this limitation by incorporating Artificial Intelligence and Machine Learning algorithms such as K-Nearest Neighbours (KNN), Deep Neural Networks (DNN), Convolutional Neural Networks (CNN), Random Forests, and Kalman Filters to improve prediction accuracy.

Beigi and Shah-Mansouri introduced a hybrid Wi-Fi and BLE localization framework in [2] using ESP32 devices with Kalman Filtering and Recurrent Neural Networks. Their work achieved promising localization accuracy but did not incorporate cybersecurity mechanisms to protect wireless communication. Similarly, recent IEEE Access and arXiv studies have demonstrated the effectiveness of AI-assisted RSSI fingerprinting for indoor localization. Despite these improvements, most existing solutions prioritize positioning accuracy while paying limited attention to authentication, encryption, anomaly detection, and secure deployment in real-world environments.

In [3] Long et al. proposed EP-3DLoc, a Wi-Fi RSSI-based framework for reliable indoor localization in multi-building and multi-floor environments. The framework combines Environment-Invariant Feature-based Data Completion (EIC) to address sparse RSSI fingerprint data with Position-Related feature-based Localization (PRL) to model the hierarchical relationship between buildings, floors, and coordinates. The approach was evaluated on multiple public datasets and the self-developed HZAU Indoor Loc dataset, achieving high localization accuracy and demonstrating robustness to signal interference.

In [4] Hussein et al. proposed an indoor object localization approach that combines Morphological Component Analysis (MCA) for RSSI noise reduction, statistical and entropy-based feature extraction, Minimum Redundancy Maximum Relevance (MRMR) for feature selection, and an LSTM-based neural network for location classification. The proposed method was designed to improve localization accuracy in challenging indoor environments affected by signal noise and interference. Experimental results reported a 97.5% classification accuracy, outperforming several conventional machine-learning and deep-learning approaches.

## Literature Review Summary Table

The columns in the table depend upon your problem and should be specific to your project.

Table 1: History of Previous WIFI based projects

.

| Study / Model | Technology Used | AI / ML Technique | Security Features | Limitations | Sentinel-X Improvement |
| --- | --- | --- | --- | --- | --- |
| Beigi & Shah-Mansouri (2023) | Wi-Fi + BLE (ESP32) | Kalman Filter, KNN, RNN | No dedicated security | Focuses mainly on localization accuracy; no cybersecurity or dashboard integration. (arXiv) | Sentinel-X adds secure communication, anomaly detection, web dashboard, and intelligent asset monitoring. |
| Adaptive Wi-Fi Fingerprinting (2024) | Wi-Fi RSSI | Deep Neural Network (DNN) | Not addressed | Requires complex dataset creation and high computational resources. (arXiv) | Sentinel-X uses simpler, low-cost AI techniques suitable for real-time deployment on university infrastructure. |
| Wi-Fi Fingerprinting Datasets Review (IEEE Access, 2024) | Wi-Fi RSSI | Fingerprinting-based methods | Not addressed | Highlights lack of standardized datasets and sensitivity to environmental changes. (Royal Holloway Research Portal) | Sentinel-X combines RSSI calibration and AI-assisted localization to improve robustness in dynamic environments. |
| Comparative Analysis on Wi-Fi Indoor Positioning (IEEE ICTC, 2024) | Wi-Fi RSSI | AI-enhanced Fingerprinting | Limited discussion | Focuses on localization performance without integrating security mechanisms. (ETRI Knowledge Sharing Platform) | Sentinel-X integrates localization, cybersecurity, and asset tracking into a unified system. |
| ML-Based Wi-Fi/BLE Indoor Positioning Survey (2024) | Wi-Fi & BLE | KNN, CNN, DNN, SVM, Random Forest | Generally not considered | Survey concludes that most research emphasizes accuracy while overlooking security and deployment cost. (PMC) | Sentinel-X emphasizes affordability, security, scalability, and practical deployment using ESP32 devices. |
| Proposed Sentinel-X | Wi-Fi (ESP32) | Kalman Filter, Fingerprinting (KNN), Isolation Forest | AES-256, TLS, JWT, Replay Protection, Rogue AP Detection | Designed as a prototype for controlled indoor environments | Provides an integrated solution combining IoT, AI, Wireless Networking, and Cybersecurity for secure and intelligent indoor asset tracking. |

In Table 1 the summary of various wifi based projects is presented in which

## Research Gap

Most existing systems focus on localization accuracy while overlooking cybersecurity and intelligent anomaly detection. Sentinel-X integrates AI-assisted localization and cybersecurity into a single low-cost indoor positioning system.

## Problem Statement

Indoor environments such as universities and laboratories contain valuable equipment and assets that require continuous monitoring and secure management. Existing tracking methods are either manual, expensive, or provide limited real-time visibility.

Although several indoor positioning technologies exist, many require costly hardware, additional infrastructure, or fail to provide adequate protection against cybersecurity threats.

Moreover, Wi-Fi-based systems often suffer from inaccurate position estimation due to RSSI fluctuations caused by environmental interference. The absence of intelligent anomaly detection further limits their effectiveness in detecting unauthorized asset movement or suspicious activities.

Therefore, there is a need for a secure, intelligent, and cost-effective indoor positioning system that improves localization accuracy while protecting communication and location data from common wireless attacks.

# Requirements and Design

## Requirements

### Functional Requirements

| ID | Functional Requirement |
| --- | --- |
| FR-01 | The system should be able to provide secure user authentication and role-based access control for authorized users. |
| FR-02 | The system should be able to provide registration and management of assets and ESP32 tracking devices. |
| FR-03 | The system should be able to collect and process indoor positioning data from ESP32 devices to estimate the real-time location of registered assets. |
| FR-04 | The system should be able to provide a real-time dashboard for monitoring asset locations, movement history, and system status. |
| FR-05 | The system should be able to detect and report abnormal or unauthorized asset movement through alerts and system notifications. |

### Non-Functional Requirements

### Hardware and Software Requirements

The Sentinel-X system integrates Internet of Things (IoT), Artificial Intelligence (AI), wireless networking, and cybersecurity technologies to provide a secure indoor positioning and intelligent asset tracking solution. The hardware and software requirements have been selected to ensure reliable real-time localization, secure communication, scalability, and ease of future enhancements. The proposed system utilizes ESP32-based tracking devices and anchor nodes for RSSI data collection, while a web-based application processes, stores, and visualizes location information.

Hardware Requirements

| Component | Specification | Purpose |
| --- | --- | --- |
| ESP32 Dev Kit Boards | ESP32-WROOM-32 (5–8 units) | Used as tracking tags and anchor nodes for Wi-Fi RSSI data collection. |
| Wi-Fi Router | IEEE 802.11 b/g/n compatible | Provides wireless communication between ESP32 devices and the server. |
| Computer/Laptop | Intel Core i5 (8th Generation or above), 8 GB RAM, 256 GB SSD | Used for software development, AI model training, and dashboard management. |
| Power Supply | 5V USB Adapter or Power Bank | Powers ESP32 development boards during deployment and testing. |
| Networking Equipment | Ethernet cables and network switch (optional) | Supports stable communication within the local network. |
| Smartphone (Optional) | Android 10 or above | Used for monitoring and future mobile application testing. |

Software Requirements

| Software | Version/Technology | Purpose |
| --- | --- | --- |
| Operating System | Windows 11 / Ubuntu 22.04 LTS | Development and deployment platform. |
| Arduino IDE | Latest Stable Version | Programming and uploading firmware to ESP32 devices. |
| Visual Studio Code | Latest Version | Source code development and debugging. |
| Python | Version 3.11 or later | AI model development, backend services, and data processing. |
| Flask / FastAPI | Latest Stable Version | RESTful API development and backend implementation. |
| React.js | Latest Stable Version | Development of the web-based monitoring dashboard. |
| PostgreSQL | Version 15 or later | Storage of asset information, RSSI data, user accounts, and movement history. |
| MQTT Broker (Mosquitto) | Latest Stable Version | Lightweight communication between ESP32 devices and the backend server. |
| Scikit-learn | Latest Version | Implementation of machine learning algorithms such as KNN and Isolation Forest. |
| Git & GitHub | Latest Version | Version control and collaborative software development. |
| Postman | Latest Version | API testing and validation. |
| Draw.io / Lucidchart | Latest Version | Creation of system architecture, UML, and workflow diagrams. |

Minimum System Requirements

| Requirement | Minimum Specification |
| --- | --- |
| Processor | Intel Core i5 |
| RAM | 8 GB |
| Storage | 256 GB SSD |
| Network | Wi-Fi |
| Internet | Required for software installation, updates, and GitHub integration |

## Proposed Methodology

The proposed methodology for the Sentinel-X system follows a structured Software Development Life Cycle (SDLC) integrated with Artificial Intelligence (AI), Internet of Things (IoT), and Cybersecurity principles. The methodology is divided into several phases, beginning with requirement analysis and ending with system evaluation. This systematic approach ensures that the developed system is reliable, secure, scalable, and capable of providing accurate real-time indoor positioning and intelligent asset tracking.

The development process begins with an extensive literature review to understand existing indoor positioning technologies, localization techniques, AI algorithms, and cybersecurity mechanisms. Based on the identified research gap, system requirements are analysed and translated into functional and non-functional requirements. A prototype architecture is then designed using ESP32 devices, Wi-Fi communication, MQTT messaging, AI-based localization, and a web-based monitoring dashboard.

During implementation, ESP32 tracking tags periodically transmit Wi-Fi beacon signals that are captured by multiple ESP32 anchor nodes. The Received Signal Strength Indicator (RSSI) values collected by these anchor nodes are transmitted to the backend server through an MQTT broker. Before localization, the RSSI values undergo preprocessing using calibration and Kalman Filtering to reduce environmental noise and signal fluctuations.

The processed RSSI data is then passed to the localization engine, where a fingerprinting-based K-Nearest Neighbors (KNN) algorithm estimates the current position of the tracked asset or user. In parallel, an Isolation Forest algorithm analyzes movement patterns to identify abnormal behavior such as unauthorized asset movement or suspicious location changes. These AI techniques improve localization accuracy while enhancing the overall security of the system.

To protect communication between IoT devices and the server, Sentinel-X incorporates multiple cybersecurity mechanisms, including AES-256 encryption, Transport Layer Security (TLS), JSON Web Token (JWT) authentication, replay attack protection, and role-based access control (RBAC). These mechanisms ensure confidentiality, integrity, and authenticity of transmitted data while preventing unauthorized system access.

The estimated locations, movement history, and anomaly alerts are stored in a PostgreSQL database. A web-based dashboard developed using React.js and Flask/FastAPI provides administrators with real-time visualization of asset locations, historical movement logs, system status, and security alerts. Finally, the prototype is evaluated using performance metrics such as localization accuracy, response time, system reliability, communication latency, and anomaly detection accuracy to determine whether the project objectives have been achieved.

Methodology Phases

| Phase | Activities | Expected Output |
| --- | --- | --- |
| Literature Review | Study existing indoor positioning systems, AI techniques, and cybersecurity mechanisms | Identification of research gap |
| Requirement Analysis | Define functional and non-functional requirements | Software Requirement Specification |
| System Design | Design architecture, database, and communication flow | System Architecture and UML diagrams |
| Hardware Setup | Configure ESP32 tracking tags, anchor nodes, and Wi-Fi network | Operational IoT hardware environment |
| Data Collection | Collect RSSI measurements from ESP32 devices | RSSI dataset |
| Signal Processing | Apply RSSI calibration and Kalman Filtering | Filtered RSSI values |
| AI Localization | Estimate indoor positions using KNN fingerprinting | Accurate location estimation |
| Anomaly Detection | Detect suspicious movement using Isolation Forest | Security alerts and anomaly reports |
| Security Implementation | Implement AES-256, TLS, JWT, and RBAC | Secure communication and authentication |
| Dashboard Development | Develop web interface for monitoring and management | Real-time monitoring dashboard |
| Testing and Evaluation | Measure localization accuracy, latency, and reliability | Performance evaluation report |

References:

1..Bellavista-Parent, V., Torres-Sospedra, J., & Perez-Navarro, A. (2022). Comprehensive analysis of applied machine learning in indoor positioning based on wi-fi: An extended systematic review. Sensors, 22(12), 4622.

2.. Beigi, K., & Shah-Mansouri, H. (2024, April). An intelligent indoor positioning algorithm based on Wi-Fi and Bluetooth Low Energy. In 2024 IEEE Wireless Communications and Networking Conference (WCNC) (pp. 1-6). IEEE.

3.. W. Long et al., “Reliable Indoor Localization in Multi-Building Environments: Leveraging Environment-Invariant and Position-Related Features,” IEEE Internet of Things Journal, 2025.

4.. H. A. Hussein, M. Hamidkhani, N. K. Bachache, and M. Soltan Aghaei, “Improving the Accuracy of Indoor Object Localization in the Internet of Things Using Recurrent Neural Networks (LSTM) in Challenging Environments,” IEEE Access, vol. 13, 2025, doi: 10.1109/ACCESS.2025.35814